## LONG-RANGE WIRELESS STARTER KIT

Specifications

Long-range wireless starter Kit is pack of

- 1 x Single Channel LoRa Gateway

- 1 x (LoRa Shield + Arduino UNO)

- 1 x (LoRa/GPS Shield + Arduino UNO)

- 1 x flame sensor

- 1 x Relay

- 1 x photosensitive sensor

- 1 x Buzzer

- 1 x Ultrosonic Sensor

- 1 x DHT11 Temperature and Humidity Sensor

- 20 x dupont wire (male to male)

- 20 x dupont wire (female to female)

- 20 x dupont wire (female to male)

Single Channel LoRa Gateway

- Processor: 400MHz, 24K MIPS

- Flash: 16MB ; RAM: 64MB

- MCU: ATMega328P

- 10M/100M RJ45 Ports x 2

- WiFi : 802.11 b/g/n

- LoRa Wireless

- Power Input: 12V DC

- USB 2.0 host connector x 1

- USB 2.0 host internal interface x 1

- 1 x LoRa Interface

- Open Source OpenWrt system

- Low power consumption

- Firmware upgrade via Web

- Software upgradable via network

- Flexible protocol to connect to IoT servers

- Auto-Provisioning

- Built-in web server

- Managed by Web GUI, SSH via LAN or WiFi

- Internet connection via LAN, WiFi and USB

- Failsafe design provides robustly system

- 1 x SX1276/SX1278 LoRa module

- Limited support in LoRaWAN/ Support Private LoRa protocol

- Support upto 300 nodes

- LoRa band available at 865-867 MHz

- Max range in LoRa: 3~10 km

- Up to 14.4Mbps downlink and 5.76Mbps uplink data rates

- Bullet-proof front end: IIP3 = -12.5 dBm.

- Excellent blocking immunity.

LoRa Shield

- 168 dB maximum link budget.

- +20 dBm - 100 mW constant RF output vs.

- +14 dBm high efficiency PA.

- Programmable bit rate up to 300 kbps.

- High sensitivity: down to -148 dBm.

- Bullet-proof front end: IIP3 = -12.5 dBm.

- Excellent blocking immunity.

- Low RX current of 10.3 mA, 200 nA register retention.

- Fully integrated synthesizer with a resolution of 61Hz.

- FSK, GFSK, MSK, GMSK, LoRaTM and OOK modulation.

- Built-in bit synchronizer for clock recovery.

- Preamble detection.

- 127 dB Dynamic Range RSSI.

- Automatic RF Sense and CAD with ultra-fast AFC.

- Packet engine up to 256 bytes with CRC.

- External Antenna via I-Pex connector

LoRa GPS Shield

- LoRa Shield basic Spec mentioned above along with following GPS Spec

- Power Acquisition: 25mA, Power Tracking: 20mA.

- Compliant with GPS, SBAS.

- Programmable bit rate up to 300 kbps.

- Serial Interfaces UART: Adjustable 4800~115200 bps, Default: 9600bps.

- Update rate: 1Hz (Default), up to10Hz.

- Protocols: NMEA 0183, PMTK.

- Horizontal Position Accuracy: Autonomous <2.5 m CEP.

- TTFF@-130dBm with EASY™: Cold Start <15s, Warm Start <5s,Hot start <1s;TTFF@-130dBm.

- Without EASY™: Cold Start <35s, Warm Start <30s,Hot Start <1s.

- Timing Accuracy: 1PPS out 10ns，Reacquisition Time <1s.

- Velocity Accuracy Without aid <0.1m/s, Acceleration Accuracy Without aid 0.1m/s².

- Sensitivity Acquisition -148dBm，Tracking -165dBm，Reacquisition -160dBm.

- Dynamic Performance Altitude Max.18000m，Maximum Velocity Max.515m/s, Maximum Acceleration 4G.

- L1 Band Receiver (1575.42MHz) Channel 22 (Tracking) /66 (Acquisition).