INDUSTRIAL LoRaWAN SENSOR NODE

Description

The UC11-N1 is a fully integrated, battery powered LoRaWAN node with multiple communication interfaces for connecting to a wide range of external sensors. With quick installation and easy maintenance, the UC11-N1 is designed to work in harsh environments for remote monitoring and collecting data from sensors.

Specifications

- IP67 waterproof for harsh environments

- Multiple interfaces including serial, I/O, CAN and I2C

- Standard LoRaWAN v1.0/v1.0.2 protocol support

- Ursalink Cloud compliant

- Multiple power supply option: battery, DC or solar

- 5 years life under LoRaWAN Class A mode

- Frequency: IN865

- Tx Power: 20dBm

- Sensitivity: -147dBm @300bps

- Mode: OTAA

- Interface: 2×GPIO, 1×RS232/RS485/CAN/I2C switchable, 1×3.3 V output, 1×5/9/12 V output switchable, 2×Analog input

- Operating Temperature: -20°C to +70°C

- Ingress Protection: IP67

- Dimension: 120.1 × 120.1× 55.4 mm (4.73 × 4.73 × 2.18 in)

- Power Input: 	1. 19000mhA replaceable Li-SOCL2 battery

2. 5-24 VDC with 4000mhA battery backup

3. Solar powered with 8000mhA battery